# Treadmill Step Counter Controller

User Manual

StampPLC single treadmill controller
Operator instructions for normal use and basic troubleshooting

Document version 1.1
16 September 2026
For firmware revision 5

# Purpose

This controller counts treadmill steps, stops the treadmill when a selected target is reached, retains the count through a power interruption, and shows operating status on the local display. The controller enables the treadmill's existing run permit; the operator still starts and operates the treadmill using its normal controls.

Operator safety. Use the mushroom E-stop whenever immediate stopping is required. The E-stop's hardwired NC contact opens the treadmill stop or permit circuit independently of the software. Do not bypass the treadmill's original safety key, guards, interlocks, or controls.

# Quick Start

Release the mushroom E-stop. Confirm EST OK. If a previous result or fault remains, inspect the machine and record the count, then hold C for three seconds and release to reset.

Briefly press A, B, or C to select the desired target shown along the lower portion of the screen.

Hold B for at least one second, then release it. The controller changes to ARMED, relay R1 closes, the red indicator turns off, and the green indicator turns on.

Start the treadmill with its normal controls. The controller changes to RUNNING after it receives the first optical pulse, and the displayed count increases.

When the count reaches the target, the controller opens relay R1, displays TARGET REACHED, activates the red indicator and buzzer, and freezes the completed count.

After the treadmill has stopped, hold C for at least three seconds and release it to clear the completed count.

# Controls

| Control | Short press | Press and hold |
| --- | --- | --- |
| A | Selects target A, normally 10,000 steps | No hold function |
| B | Selects target B, normally 50,000 steps | Hold 1 second and release to arm, pause, or resume |
| C | Selects target C, normally 100,000 steps | Hold 3 seconds and release to reset while stopped |
| Mushroom E-stop | Not applicable | Press to stop; twist or pull as marked to release |

Button timing. Actions occur when the button is released. A B hold shorter than one second selects preset B. A C hold between one and three seconds performs no action.

# Indicator Light and Buzzer

| Indication | Meaning |
| --- | --- |
| Green | The treadmill permit is enabled and the controller is ARMED or RUNNING. |
| Red | The permit is disabled. This is normal while idle, stopped, complete, or faulted. |
| Buzzer | Sounds briefly when a stop or fault is registered. |

# Display Guide

The center of the display shows the controller state, accumulated step count, selected target, and the three preset values. Status indicators around the perimeter show live inputs and commanded relay states.

## Top indicators

| Display | Meaning |
| --- | --- |
| OPT OFF | Optical input is inactive according to the configured polarity. |
| OPT ON | Optical input is active. A debounced inactive-to-active transition counts a step while armed or running. |
| OPT ERR | The PLC input subsystem is not responding. |
| EST OK | The E-stop auxiliary input is in the released state. |
| EST STOP | The E-stop has been pressed or its auxiliary input is active. |
| EST ERR | The PLC input subsystem is not responding. |
| 485 OFF | RS-485 is not enabled in this standalone version. |

## Controller states

| State | Meaning |
| --- | --- |
| IDLE | Stopped and ready for target selection or arming. |
| ARMED | Permit enabled; waiting for the first optical pulse. |
| RUNNING | Pulses are being counted. |
| PAUSED | Permit disabled by the B button or web control; count retained. |
| TARGET REACHED | Target met; permit disabled and count frozen until reset. |
| STOPPED | Stopped by the E-stop or another stop command; count retained. |
| FAULT | A condition prevents operation. See Troubleshooting. |

## Bottom relay indicators

| Display | Commanded function |
| --- | --- |
| R1 | Treadmill run permit |
| R2 | Green indicator |
| R3 | Red indicator |
| R4 | External buzzer |

R1 through R4 show the software command sent to each relay. They do not confirm the physical position of the relay contacts or the state of the connected equipment.

The optical badge also flashes for approximately 350 milliseconds after a detected pulse. Automatic screen dimming, wake-only button presses, and pixel shifting are not enabled in this firmware.

# Operating Procedures

## Selecting a target

While stopped, briefly press A, B, or C. The selected target appears after TARGET on the display. Selecting a different target does not clear the accumulated count. Target selection is blocked while armed, running, completing a coast-down count, or holding a completed result.

## Arming and starting

1.  Confirm EST OK and verify that no fault is displayed.

2.  Select the required target.

3.  Hold B for at least one second and release it. Confirm ARMED, green indication, and R1 ON.

4.  Start the treadmill using its normal controls. Confirm that the count advances and the state changes to RUNNING.

## Pausing and resuming

Hold B for at least one second and release it while armed or running. The controller opens the permit and changes to PAUSED. Once the treadmill and optical mechanism are settled, hold B again for at least one second and release it to re-arm. Use the treadmill's normal control to restart it if required.

## Using the E-stop

Press the mushroom E-stop for an immediate stop. Its hardwired NC contact opens the treadmill stop or permit circuit, while its separate auxiliary contact tells the StampPLC to display EST STOP and retain the count. Correct the condition that caused the stop, release the E-stop, and verify EST OK before resetting or re-arming.

## Completing and resetting a run

At the target, the controller disables the permit and freezes the count until reset. Record the result before clearing it. After all motion has stopped, release the E-stop, hold C for three full seconds, and release. A successful reset shows Reset OK - stopped, clears the count and fault, silences the external buzzer, and returns to IDLE. The target stays unchanged and the treadmill remains stopped. Hold B separately to arm another run.

A reset is rejected if the controller is armed or running, the E-stop is active or still settling, the input subsystem cannot be read, or the FRAM cannot save and verify the reset. Recent optical pulses no longer block reset. Arming and local preset selection still require three seconds without an optical transition.

Reset retries communication with the inputs and, when unavailable, the FRAM. It clears a recoverable fault only after those checks and the saved reset succeed. If saving fails, the displayed count is restored and the controller stays faulted. Reset messages appear on the screen for eight seconds; the web dashboard retains the latest reset message.

After a pause or non-completion stop, pulses during coast-down may still increase the count. At TARGET REACHED the completed count stays frozen. Reset clears any coast-down counting.

## After a power interruption

The FRAM restores the saved count and target. The controller does not automatically restore the treadmill permit. If power was lost while armed or running, the controller enters FAULT to require inspection before another run. A pulse occurring during the instant of power loss may not be retained.

# Web Interface

The display alternates between the controller address and the local button reminder when no error or reset message takes priority. Enter http:// followed by that address in a browser. Use the credentials supplied by the installer; unchanged defaults are listed below.

| Setting | Default |
| --- | --- |
| Controller Wi-Fi name | flux- followed by the controller identifier |
| Controller Wi-Fi password | fluxcapacitor |
| Direct connection address | http://192.168.4.1 |
| Web username | admin |
| Web password | change-this-password |

## Connecting to an existing network

1.  Stop the run and wait for all motion to stop. Saving or clearing Wi-Fi settings restarts the controller; the settings page does not block this during a run.

2.  For first setup, connect your phone or computer to the controller's flux- network. Open http://192.168.4.1 and sign in.

3.  Open Wi-Fi settings. Enter the network name and password, then save. Use a compatible 2.4 GHz personal Wi-Fi network. A blank password is for an open network; enterprise username-based login is not supported by this sketch.

4.  After the restart, connect your phone or computer to the same network and open the controller's new address shown on its display.

The controller remembers these settings across power cycles. If it cannot connect after about 15 seconds, it starts its fallback flux- network and continues retrying the saved network about every 30 seconds. Reconnect directly to http://192.168.4.1 to correct the credentials. Forgetting the saved network restarts the controller in access-point mode. Resetting the count does not erase Wi-Fi settings.

Use the web interface on a trusted network. The installer changes the default access-point and web passwords in the sketch; the Wi-Fi settings form changes only the network the controller joins.

## Web controls

| Web control | Function |
| --- | --- |
| Pause and stop | Opens the permit and retains the count. |
| Stop | Stops the current operation and retains the count. |
| Acknowledge | Silences the external buzzer only. Does not clear a fault or reset the count. |
| Reset count | Clears the count when all reset conditions are satisfied. |
| Set target | Sets 1 to 1,000,000,000 steps while stopped and without a completed result. Does not change A/B/C presets or clear the count. |

The web interface does not replace the mushroom E-stop or the treadmill's normal controls. Loss of Wi-Fi does not stop local pulse counting or disable the hardwired E-stop circuit.

Arming is local only: use a one-second B hold and release. The web interface has no Arm control. After a web reset, check the displayed count, state, and latest reset message to confirm the result.

# Troubleshooting

| Reset message | Action |
| --- | --- |
| Reset OK - stopped | Reset succeeded. Count is zero; arm separately when ready. |
| Reset: stop run first | Pause or stop before resetting. |
| Reset: release E-stop | Release the mushroom, confirm EST OK, then retry. |
| Reset: input IO fault | Input communication failed. Have the installer check the controller if a retry fails. |
| Reset: FRAM bus fault | FRAM bus could not initialize. Check with the installer. |
| Reset: FRAM missing | FRAM did not respond. Have its connection and power checked. |
| Reset: FRAM save failed | Reset could not be verified. Count remains displayed; repair storage before another run. |

## Other symptoms

| Symptom | Check |
| --- | --- |
| BENCH LOCK | The commissioning lock remains enabled in firmware. The installer must verify the wiring and load the production configuration. |
| C does not reset | Stop or pause, release the E-stop, hold C for 3 seconds and release. Read the reset message; a C hold between 1 and 3 seconds does nothing. |
| Count does not advance | Confirm RUNNING or ARMED, observe OPT OFF and OPT ON as the interrupter changes, and check sensor alignment and wiring. |
| Count advances twice | Check for a loose or vibrating flag, marginal alignment, or multiple optical transitions per mechanical step. |
| Will not arm | Check EST OK, FRAM status, input status, target greater than count, and that no optical transition occurred during the last 3 seconds. |
| FAULT after startup | Power may have failed during a run, or the FRAM/input subsystem failed. Inspect the treadmill and count before resetting. |
| Pulse timeout | No first pulse arrived within 60 seconds of arming, or no pulse arrived for 15 seconds while running. Inspect the sensor and treadmill, then reset before re-arming. |
| Red light while idle | Normal. Red means the treadmill permit is disabled. |
| R1 ON but treadmill will not run | Check the E-stop, original treadmill controls, safety key, and permit-loop wiring. R1 only enables the permit. |

# Care and Inspection

Before use, check the enclosure, cable security, sensor mounting, and target flag.

Test the E-stop regularly and confirm that it stops the treadmill even if the StampPLC is rebooting or unresponsive.

Keep the optical slot clean. Stop using the counter if counts become erratic; have sensor alignment and mounting checked.

Have the installer perform electrical repairs with power isolated.